GoWarrior FTool

V1.0

User Manual

Copyright Statement

ALi Corporation makes no representations or warranties with respect to the accuracy or completeness of the contents of this document. No license, whether express, implied, arising by estoppel or otherwise, to any intellectual property right is granted by this document.

ALi Corporation also reserves the right to make changes to these specifications and product description at any time without notice.

Contact your local sales office to obtain the latest specifications before placing your order.

Third-party brands and names mentioned in this publication are for identification purpose only and may be the property of their respective owners.

Microsoft®, and Windows® are registered trademarks of Microsoft Corporation in the United States and other countries.

Linux® is a registered trademark of Linus Torvalds.

Supply of this Implementation of Microsoft Corporation and Linus Torvalds does not convey a license nor imply a right under any patent, or any other industrial or intellectual property right of Microsoft Corporation and Linus Torvalds, to use this Implementation in any finished end-user or ready-to-use final product. It is hereby notified that a license for such use is required from Microsoft Corporation and Linus Torvalds.

Copyright © 2015 ALi Corporation. All rights reserved.

Table of Contents

Preface	1

Overview	1

Audience	1

Application Products	1

Conventions	2

Acknowledgement	3

1	Environment Preparation	4

2	Flash Burning	6

2.1	Connecting to Platform	6

2.2	Opening Profile	8

2.3	Selecting Partitions to Burn	9

2.4	Starting Burning	11

2.5	Finishing Burning	11

3	Making A Burning File Package	13

4	Flash Dumping	15

4.1	Connecting to Platform	15

4.2	Opening Profile	15

4.3	Selecting Items to Dump	15

4.4	Starting Dumping	16

4.5	Finishing Dumping	17

5	Modifying MAC Address of Platform	18

5.1	Connecting to Platform	18

5.2	Opening Profile	18

5.3	Modifying MAC Address	18

6	Partition Burn Check	20

Revision History	21

List of Tables

Table 1. Typographical Conventions	2

Table 2. Symbol Conventions	2

Table 3. Revision History	23

List of Figures

Figure 1. Partition Information	5

Figure 2. Sample Cable and Adapter	6

Figure 3. Sample Demo Board	6

Figure 4. Connect Platform to PC via USB Cable	7

Figure 5. Upgrade Button	7

Figure 6. Connect to Platform	8

Figure 7. Open Profile	9

Figure 8. Partition Options	10

Figure 9. Select Partitions to Burn	11

Figure 10. Start Burning Flash	12

Figure 11. Burning Completed	13

Figure 12. Archive Name and Parameters	15

Figure 13. Select Items to Dump	17

Figure 14. Start Dumping	18

Figure 15. Dumping Completed	19

Figure 16. Edit MAC Address	21

Figure 17. Partition Burn Check	22

Preface

- Overview

This guide mainly introduces how to use FTool tool to burn NAND Flash of ALi platform. It is organized by the following chapters:

Chapter 1: Environment Preparation

This chapter introduces the environment required by the FTool.

Chapter 2: Flash Burning

- This chapter introduces how to burn NAND Flash by using FTool.

Chapter 3: Making A Burning File Package

This chapter introduces how to make a burning file package.

Chapter 4: Flash Dumping

This chapter introduces how to dump data from NAND Flash.

Chapter 5: Modifying MAC Address in Platform

This chapter introduces how to modify the MAC address of the partition file parsed by FTool.

Chapter 6: Partition Burn Check

This chapter introduces how FTool reads data back from NAND Flash and checks them against original data.

- Audience

This manual is mainly applicable for the following engineers.

Test engineers

Software development engineers

FAE (Field application engineers)

- Application Products

The manual is applicable for ALi platforms supporting NAND Flash.

- Conventions

Typographical Conventions

| Item | Format |
| --- | --- |
| codes, keyboard input commands, file names, equations, and math | Courier New, Size 10.5 |
| Variables, code variables, and code comments | Courier New, Size, Italic |
| Menu item, buttons, tool names | Times New Roman, Size 10.5, Bold e.g. Select USB Debugging |
| Screens, windows, dialog boxes, and tabs | Times New Roman, Size 10.5, Bold Enclosed in double quotation marks e.g. Open the “Debug Configuration” dialog box |

Table 1. Typographical Conventions

Symbol Conventions

| Item | Description |
| --- | --- |
| Caution | Indicates a potential hazard or unsafe practice that, if not avoided, could result in data loss, device performance degradation, or other unpredictable results. |
| Note | Indicates additional and supplemental information for the main contents. |
| Tip | Indicates a suggestion that may help you solve a problem or save your time. |

Table 2. Symbol Conventions

- Acknowledgement

Microsoft®, and Windows® are registered trademarks of Microsoft Corporation in the United States and other countries.

Linux® is a registered trademark of Linus Torvalds.

Environment Preparation

FTool is a software tool that runs on Windows operating system. It is used to burn the NAND Flash of ALi platform and requires the following items:

FTool.exe: i.e. the burning tool

ALI.ini: This file is automatically generated at compile time. Please open it. Description of other files required by FTool.exe can also be found in this file.

Note:

As FTool is applied to both the Linux solution and Android solution, the location of generated ALi.ini and other files after compilation will differ according to different customized projects.

NandList_v2.ran: This is a NAND flash list file. It needs to be updated if a new flash is added.

sdram_C3921_QFP_1GB_1066Mbps.abs: It is the SDRAM profile. Please select a correct platform SDRAM profile.

nand_updater_loader.axf.bin: It is a file for implementing burning.

uboot_unify_1GB_training.abs: Loader

Note:

In Android system, FTool is located in this directory: AOSP/build/tools/burningtools/FTool/. When executing the “build image” command to generate Android image files, the system will simultaneously and automatically copy files in FTool directory to the AOSP/image directory where Android image files are located.

As FTool is applied to both the Linux solution and Android solution, the location of generated burning file in each partition after compilation will differ according to different customized projects.

Figure 1. Partition Information

Flash Burning

Connecting to Platform

Prepare a USB cable and an adapter as shown below.

Figure 2. Sample Cable and Adapter

Connect the platform (shown in Figure 3) to a PC using the USB cable, as shown in Figure 4. Press and hold the “Burn NAND” button, and connect platform to PC via USB cable.

Figure 3. Sample Demo Board

Figure 4. Connect Platform to PC via USB Cable

Run FTool.exe.

If the following screen appears, it indicates successful connection. If not, please try again.

Figure 6. Connect to Platform

Opening Profile

When FTool is opened, the profile ALI.ini in the current directory will be parsed automatically. Other profiles can also be opened.

Please pay attention to the size of partition list in ALI.ini. The partition size should be greater than the size of burned file.

[PARTITION1]

NAME = bootbak

SIZE = 0x800000

FILE = uboot_unify_1GB_training.abs

The TOTAL-SIZE of partition should be less than the capacity of NAND Flash.

For example, the capacity of NAND Flash selected should be greater than 0x24400000 bytes, or the partition list in ALI.ini needs to be reconfigured.

[PARTITION-COUNT]

COUNT = 9

TOTAL-SIZE = 0x24400000

Figure 7. Open Profile

Selecting Partitions to Burn

You can either choose Select All to burn the whole Flash or choose Select None to cancel all the partitions selected.

Figure 8. Partition Options

You can also select one or more partitions to burn, or check Erase All Partitions to erase the whole Flash.

Figure 9. Select Partitions to Burn

Starting Burning

Figure 10. Start Burning Flash

Finishing Burning

When the following screen appears, it indicates successful burning.

Figure 11. Burning Completed

Making A Burning File Package

FTool supports burning the single file in ZIP archive format.

There are two methods to make the burning file package.

- Use Linux zip command

Put the files (exclusive of files related to FTool) to be zipped in the same directory, and execute the following command.

user@shsa02:~/work/SDK6.0ha.1.1_ACAS1.1_20140507$ zip -0 ALi.abp *

Use Windows zip utility for file compression (WinZIP/WinRAR).

Figure 12 illustrates the options that need to be configured for making a zip package. Please pay attention to the items circled in blue. The files will be packetized to produce ALi.abp file.

Note:

When the second method is adopted, if some files in the storage package need to be updated, do not drag the files to the compression tool interface directly .The proper way to do this is to unzip the storage package to some directory, use the new files to overwrite the original files, and recompress the files to generate a new storage package by making configurations indicated in Figure 12.

Figure 12. Archive Name and Parameters

Flash Dumping

Connecting to Platform

It is the same with that in 2.1 Connecting to Platform.

Opening Profile

It is the same with that 2.2 Opening Profile.

Generally, the opened profile should match the partition list in the platform. Otherwise, the data dumped by partition may be incorrect.

Selecting Items to Dump

- Dump a partition

Select a partition in the partition list, and the address and length of this partition will be automatically calculated in the Dump Tool Column..

- Dump a partition segment

If you need to dump the entire NAND Flash or dump the specified data address and length, please enter the address and length in the Address and Length fields of the Dump Tool area.

Dumping does not skip the bad blocks by default. If you wish to skip the bad blocks, please select Skip Bad Block.

Figure 13. Select Items to Dump

Starting Dumping

Figure 14. Start Dumping

Finishing Dumping

Figure 15. Dumping Completed

Modifying MAC Address of Platform

Connecting to Platform

It is the same with that in 2.1 Connecting to Platform.

Opening Profile

It is the same with that 2.2 Opening Profile.

Modifying MAC Address

If a burn file contains the deviceinfo file, FTool will automatically resolve MAC address from the deviceinfo file and display it on the screen, as shown in Figure 16. You can edit the MAC address by clicking on MAC edit box.

Figure 16. Edit MAC Address

Note:

FTool will automatically detect whether MAC address needs to be modified. During burning, FTool will compare the MAC address info in the edit box of the screen with that saved in the deviceinfo partition. Once the address difference is detected, it will automatically tick the deviceinfo partition and burn MAC.

Partition Burn Check

Upon completion of partition burning, FTool will read back the partition data from Nand Flash and compare it with the original data to ensure successful burning. As this process takes a relatively long time, the Burn Check partitions option on the interface is provided to support reading back or not reading back data for confirmation.

FTool defaults to check Burn Check Partitions. If you wish to cancel the data confirmation process, you only need to uncheck Burn Check Partitions.

Figure 17. Partition Burn Check

Revision History

| Date | Revision | Changes |
| --- | --- | --- |
| May 15, 2015 | - v2.9 | Added detailed description about how to connect a platform to PC in 2.1 Connecting to Platform. |
| Feb. 3, 2015 | - v2.9 | Replaced FTool screenshots throughout this manual. |
| Jan. 8, 2015 | - v2.5 | Initial release |

Table 3. Revision History

www.alitech.com

USA Office

Tel: +1-480-882 8639

Fax: +1-480-491 0202

Europe Office

Tel: +41-22-816-1900

Fax: +41-22-816-1949

Korea Office

Tel: +82-2-2043-3601

Fax: +82-2-2043-3602

Shenzhen Office

Tel: +86-755-2519-5788

Fax: +86-755-2519-5393

Headquarters

Tel: +886-2-8752-2000

Fax: +886-2-8751-1001